4. Model Description:

Action:

This section contains the essential building details for evaluating the controller's performance. The numerical study focused on a 20-story building equipped with an active cable system as a reference. This nonlinear model was developed to address taller structures' complicated challenges in responding to seismic forces and nonlinear behavior. This model takes stiffness variations, local failures, and dynamic interactions between structural components, which require advanced control systems to improve structural stability. This standard reference structure explicitly developed for seismic control testing would provide a reliable platform for analyzing and comparing different control algorithms. The main advantages of the model are listed below:

1. Standardization of Evaluations: The researchers' assessments are based on uniform parameters, which are ensured using a benchmark building with standard specifications. This improves the comparability of results between different studies and makes the results more reliable.

2. Realistic Simulation of Nonlinear Behavior: The building's nonlinear properties provide a closer approximation to real conditions, especially for the beam and column elements. This enables the system to test the control systems more accurately under seismic conditions and reflects urban structures' actual complexity and dynamic behavior.

3. Model Simplicity: Using benchmark buildings with pre-defined models reduces the time and costs of simulating complex structures. This method helps researchers concentrate on developing and improving control algorithms without individual structural modeling.

4. Increased Credibility of Results: Results obtained using benchmark structures are more reliable and trustworthy since the scientific communities have already studied them.

5. The Global Comparison Possibility: Benchmark structures make research results comparable, which encourages researchers to share their findings and experiments.

6. Development of Optimizations and Innovations: The standardized nature of benchmark buildings motivates researchers to improve their control algorithms to achieve better performance metrics than standard models.

7. Facilitating the Process of Results Reproduction: Because benchmark buildings are publicly accessible, they facilitate other scientists' reproduction of research results. This is important to validate the results, ensure their accuracy and advance the field of earthquake protection.

8. Integration of Practical Experience and Knowledge: To help researchers, benchmark structures are designed with practical knowledge to evaluate the control system under real structural challenges. This integration of theory and practice enhances the applicability of the results.

These advantages make benchmark structures a reliable tool for evaluating and comparing control algorithms, which can improve the accuracy of research results.

And (page 31):

Figure 11 depicts the non-linear evaluation model simulator. The user incorporates SIMULINK blocks corresponding to their sensors and controllers to simulate the model and evaluate the proposed control strategy based on the following criteria.

Figure 11: SIMULINK block diagram of the vibration control simulation system.

5. Numerical Analysis:

Action: )The explanations are provided on pages 32 to 36 of the manuscript. (

To evaluate the performance of the IT2FNN-FOPID controller in comparison with other control strategies, including LQG [70], fuzzy [73], clipped-LQG control [74], semi-active MPC [74], semi-active H-∞ [74], AFOPID I [75] and AFOPID II [75], time history analyzes of the structural responses under the influence of four known earthquakes were performed. The selected earthquakes  El Centro 1940, Hachinohe 1968, Northridge 1994 and Kobe 1995  provide a robust benchmark for testing control strategies as they exhibit different seismic characteristics. These records were selected according to tectonic location, fault mechanism and historical significance to create different simulation conditions. Both parametric and seismic uncertainties were fully considered in this study. A bilinear hysteresis model is used to simulate plastic hinges and yielding structural elements to capture the nonlinear behavior of structures under significant seismic excitations. In addition, the analysis includes customized earthquake recordings at different seismic intensities to evaluate the system's response.

This method allows a detailed evaluation of the control system's performance under different seismic conditions and justifies the complexity associated with the design and analysis of the structures. By integrating realistic nonlinear behavior and varying earthquake intensities, this approach improves the reliability and robustness of design solutions by providing valuable insights into the dynamic interactions between structural responses and control strategies during unpredictable seismic events.

earthquakes characteristics Details

- The 1940 El Centro earthquake (Far-field) during the Imperial Valley seismic event in California was essential evidence of ground motion in earthquake engineering. Due to its long duration and moderate acceleration, it is a valuable benchmark for evaluating structural response in distant seismic scenarios, with a moment magnitude of about 6.9. The earthquake reached a peak ground acceleration (PGA) of 0.3483 g, making it an essential benchmark for seismic analysis. The complex energy release of this event provides invaluable data for evaluating control systems, especially for structures with prolonged natural periods of vibration. The notable amplitude and long duration of the ground-shaking event increase its suitability for studying the performance of control mechanisms under far-field seismic conditions and provide essential insights into structural responses and control strategies.

The 1968 Hachinohe earthquake in Japan is known for its significant ground shaking over a large area. It substantially affected buildings with a magnitude of about 7.8 and a PGA of 0.2294 g. The long duration of the earthquake and low-frequency content challenged the control systems in terms of durability and adaptability.

- The 1994 Northridge earthquake (near-field), which caused extensive damage to buildings and infrastructure in California with a moment magnitude of 6.7 and a PGA of 0.842g, provides important data on severe shaking near the fault, making it an essential case for near-field seismic studies. The thrust fault mechanism highlights the seismic risks of hidden faults and warns of the necessity of improved seismic design to address near-field solid ground motions. The peak ground acceleration recorded during the earthquake is characterized by vertical ground motion and forward direction effects by short duration and high intensity. This is a crucial benchmark for evaluating the control systems' ability to deal with abrupt, high-amplitude shaking.

- The 1995 Kobe earthquake (near-field), considered as Great Hanshin, struck Japan with a magnitude of 6.9 and caused significant destruction due to its complex ground motion patterns that were notable for the strong ground motions near the fault with peak ground acceleration of 0.833g near the fault with significant vertical and horizontal ground motions and forward direction. The severe impact of this earthquake has highlighted the importance of understanding the seismic effects at close range, especially in densely populated areas. It has also provided important insights for the design and retrofitting of buildings to withstand intense ground motions. The Kobe earthquake is a notable example of evaluating the performance of control systems to mitigate the severe seismic effects on buildings near the fault.

Criteria for Selecting Earthquakes

The International Association for Structural Control (IASC) has selected these ground acceleration values to provide a comprehensive assessment of the performance of control systems under different seismic conditions. The criteria for selecting these records are based on several key factors as below:

Diversity in Seismic Conditions: The selected earthquakes represent a broad spectrum of seismic conditions. Distant earthquakes (e.g., El Centro and Hachinohe 1968) are characterized by moderate intensity, low frequencies, and long-lasting ground motions that occur considerably from the fault line. In contrast, near-fault earthquakes like Northridge and Kobe are evaluated by intense and abrupt displacements, high-velocity pulses, and strong accelerations. This movement diversity allows for a better analysis of the structural control system's performance.

Data Availability: These earthquakes' well-documented and perfect ground acceleration records characteristics are necessary for performing accurate and reliable simulations and analyses.